Literature Survey on ACT-R Based Driving and Simulation Systems

Recent research in intelligent transportation and autonomous vehicles has increasingly focused on integrating cognitive models, simulation environments, and control strategies to better replicate human driving behavior and improve safety. The selected literature highlights three major directions: driving simulation systems, cognitive modeling using ACT-R, and trajectory planning/control in autonomous vehicles.

Driving simulators play a critical role in studying driver behavior and human–vehicle interaction. Li et al. (2025) conducted a thematic analysis based on expert interviews to identify key requirements for effective driving simulators. Their work emphasizes three major dimensions: participant experience, research needs, and operational requirements. The study reveals that despite technological advancements, many simulators lack usability, flexibility, and realism, which limits their adoption in research. Similarly, Riera et al. (2022) focused on motion perception in simulators, highlighting that realistic vestibular feedback is essential for improving user immersion and training effectiveness. However, limitations in motion platforms and washout algorithms lead to loss of critical motion cues, affecting realism.

To address limitations in traditional traffic modeling, Espié and Auberlet (2007) proposed ARCHISIM, a behavioral multi-agent traffic simulation model. Unlike classical mathematical models that rely on statistical laws, ARCHISIM integrates driver behavior and interaction among traffic participants. This approach enables better prediction of future traffic scenarios, although modeling complex human behavior remains challenging.

Cognitive architectures, particularly ACT-R, have been widely used to model human driving behavior. Li et al. (2018) proposed an ACT-R-based trajectory planning method for electric vehicle obstacle avoidance. Their model incorporates modules such as behavior initialization, trajectory planning, and weight adjustment, enabling dynamic adaptation to road conditions. The results demonstrate improved performance compared to optimal control and fuzzy neural methods, especially in capturing human-like decision-making.

Further enhancements to ACT-R have been explored to better represent real-world driving complexity. Liu and Wu (2007) improved ACT-R by introducing parallel processing mechanisms to simulate multitasking driver behavior. Their work addresses limitations of serial cognitive processing by incorporating subconscious and simultaneous actions, significantly improving modeling efficiency and realism.

Recent studies have also integrated machine learning with cognitive architectures. Meghdadi et al. proposed incorporating language model embeddings into ACT-R to improve semantic processing and scalability. This hybrid approach enhances prediction accuracy and reduces manual effort in encoding knowledge, demonstrating the potential of combining symbolic and data-driven methods.

In autonomous driving, trajectory tracking and control remain key challenges. Tang et al. (2023) developed a trajectory tracking control method combining ACT-R with preview tracking theory. By incorporating human cognitive behavior and real driving data, their model achieves high similarity to real driver actions. This approach improves personalization and adaptability in autonomous systems, addressing limitations of traditional control and machine learning-based models.

In summary, the literature shows a clear trend toward human-centric modeling in autonomous driving systems. While driving simulators and traffic models provide experimental platforms, ACT-R-based cognitive frameworks enable realistic representation of driver behavior. However, challenges remain in improving realism, scalability, and integration with modern AI techniques. Future research is expected to focus on hybrid approaches combining cognitive models, machine learning, and advanced simulation technologies.